HACETTEPE UNIVERSITY

DEPARTMENT OF ELECTRICAL AND ELECTRONICS ENGINEERING

ELE432 - Advanced Digital Design

HOMEWORK – II

MULTICYCLE PROCESSOR CONTROLLER

Kaan MANSUROĞLU

2210357098

Date: 24.04.2026

- Please indicate how many hours you spent on this lab. This will be helpful for calibrating the workload for next time the course is taught.

I have spent approximately 3 hours

2. Hierarchical SystemVerilog for your controller module matching the declaration given above.

module controller( input logic clk, input logic reset, input logic [6:0] op, input logic [2:0] funct3, input logic funct7b5, input logic zero, output logic [1:0] immsrc, output logic [1:0] alusrca, alusrcb, output logic [1:0] resultsrc, output logic adrsrc, output logic [2:0] alucontrol, output logic irwrite, pcwrite, output logic regwrite, memwrite ); logic [1:0] aluop; logic branch, pcupdate; // Main FSM mainfsm fsm( .clk (clk), .reset (reset), .op (op), .zero (zero), .alusrca (alusrca), .alusrcb (alusrcb), .resultsrc (resultsrc), .adrsrc (adrsrc), .aluop (aluop), .irwrite (irwrite), .pcwrite (pcwrite), .regwrite (regwrite), .memwrite (memwrite), .branch (branch), .pcupdate (pcupdate) ); // ALU Decoder aludec ad( .opb5 (op[5]), .funct3 (funct3), .funct7b5 (funct7b5), .ALUOp (aluop), .ALUControl (alucontrol) ); // Instruction Decoder instrdec id( .op (op), .ImmSrc (immsrc) ); Endmodule

module mainfsm( input logic clk, input logic reset, input logic [6:0] op, input logic zero, output logic [1:0] alusrca, alusrcb, output logic [1:0] resultsrc, output logic adrsrc, output logic [1:0] aluop, output logic irwrite, pcwrite, output logic regwrite, memwrite, output logic branch, pcupdate ); typedef enum logic [3:0] { S0_FETCH = 4'd0, S1_DECODE = 4'd1, S2_MEMADR = 4'd2, S3_MEMREAD = 4'd3, S4_MEMWB = 4'd4, S5_MEMWRITE = 4'd5, S6_EXECUTER = 4'd6, S7_ALUWB = 4'd7, S8_EXECUTEI = 4'd8, S9_JAL = 4'd9, S10_BEQ = 4'd10 } statetype; statetype state, nextstate; always_ff @(posedge clk or posedge reset) if (reset) state <= S0_FETCH; else state <= nextstate; always_comb begin case(state) S0_FETCH: nextstate = S1_DECODE; S1_DECODE: case(op) 7'b0000011, 7'b0100011: nextstate = S2_MEMADR; // lw or sw 7'b0110011: nextstate = S6_EXECUTER; // R-type 7'b0010011: nextstate = S8_EXECUTEI; // I-type ALU 7'b1101111: nextstate = S9_JAL; // jal 7'b1100011: nextstate = S10_BEQ; // beq default: nextstate = S0_FETCH; endcase S2_MEMADR: case(op) 7'b0000011: nextstate = S3_MEMREAD; // lw 7'b0100011: nextstate = S5_MEMWRITE; // sw default: nextstate = S0_FETCH; endcase S3_MEMREAD: nextstate = S4_MEMWB; S4_MEMWB: nextstate = S0_FETCH; S5_MEMWRITE: nextstate = S0_FETCH; S6_EXECUTER: nextstate = S7_ALUWB; S7_ALUWB: nextstate = S0_FETCH; S8_EXECUTEI: nextstate = S7_ALUWB; S9_JAL: nextstate = S7_ALUWB; S10_BEQ: nextstate = S0_FETCH; default: nextstate = S0_FETCH; endcase end always_comb begin // Defaults: all 0 alusrca = 2'b00; alusrcb = 2'b00; resultsrc = 2'b00; adrsrc = 1'b0; aluop = 2'b00; irwrite = 1'b0; pcupdate = 1'b0; regwrite = 1'b0; memwrite = 1'b0; branch = 1'b0; case(state) S0_FETCH: begin adrsrc = 1'b0; irwrite = 1'b1; alusrca = 2'b00; alusrcb = 2'b10; aluop = 2'b00; resultsrc = 2'b10; pcupdate = 1'b1; end S1_DECODE: begin alusrca = 2'b01; alusrcb = 2'b01; aluop = 2'b00; end S2_MEMADR: begin alusrca = 2'b10; alusrcb = 2'b01; aluop = 2'b00; end S3_MEMREAD: begin resultsrc = 2'b00; adrsrc = 1'b1; end S4_MEMWB: begin resultsrc = 2'b01; regwrite = 1'b1; end S5_MEMWRITE: begin resultsrc = 2'b00; adrsrc = 1'b1; memwrite = 1'b1; end S6_EXECUTER: begin alusrca = 2'b10; alusrcb = 2'b00; aluop = 2'b10; end S7_ALUWB: begin resultsrc = 2'b00; regwrite = 1'b1; end S8_EXECUTEI: begin alusrca = 2'b10; alusrcb = 2'b01; aluop = 2'b10; end S9_JAL: begin alusrca = 2'b01; alusrcb = 2'b10; aluop = 2'b00; resultsrc = 2'b00; pcupdate = 1'b1; end S10_BEQ: begin alusrca = 2'b10; alusrcb = 2'b00; aluop = 2'b01; resultsrc = 2'b00; branch = 1'b1; end default: begin end endcase end assign pcwrite = pcupdate ¦ (branch & zero); endmodule

module aludec( input logic opb5, input logic [2:0] funct3, input logic funct7b5, input logic [1:0] ALUOp, output logic [2:0] ALUControl ); logic RtypeSub; assign RtypeSub = funct7b5 & opb5; // TRUE for R-type subtract always_comb case(ALUOp) 2'b00: ALUControl = 3'b010; // add (lw, sw, fetch, decode) 2'b01: ALUControl = 3'b110; // sub (beq) default: case(funct3) // R-type or I-type ALU 3'b000: if (RtypeSub) ALUControl = 3'b110; // sub else ALUControl = 3'b010; // add, addi 3'b010: ALUControl = 3'b111; // slt, slti 3'b110: ALUControl = 3'b001; // or, ori 3'b111: ALUControl = 3'b000; // and, andi default: ALUControl = 3'b010; // don't care -> 0 endcase endcase endmodule

module instrdec( input logic [6:0] op, output logic [1:0] ImmSrc ); always_comb case(op) 7'b0110011: ImmSrc = 2'bxx; // R-type (don't care) 7'b0010011: ImmSrc = 2'b00; // I-type ALU 7'b0000011: ImmSrc = 2'b00; // lw 7'b0100011: ImmSrc = 2'b01; // sw 7'b1100011: ImmSrc = 2'b10; // beq 7'b1101111: ImmSrc = 2'b11; // jal default: ImmSrc = 2'b00; // don't care -> 0 endcase endmodule